CombinatorialPlugFluidics Installation and Software Instruction Manual

Ramesh Utharala - Merten Grou p, EM B L - June 201 7

# Installation Guide

**NOTE: You DO NOT need to have LabVIEW installed to run the executable, however if LabVIEW is not installed you will need to install LabVIEW Run-Time Engine 2013.  An .exe is included in the CombinatorialPlugFluidics.zip to install LabVIEW Run-Time Engine 2013.

## To Download Files

- Download CombinatorialPlugFluidics.zip and extract to the desired location.

## To Launch the CombinatorialPlugFluidics Executable

- In CombinatorialPlugFluidics directory navigate to the Executables folder.

- Double-click the CombinatorialPlugFluidics.exe application file to launch the program.

- Once opened the program will begin running immediately.  To correctly configure the software please refer to Running the CombinatorialPlugFluidics Executable.

## To Install LabVIEW Run-Time Engine 2013

- Download the run-time engine from National instruments website (https://www.ni.com/en-ca/support/downloads/software-products/download.labview-runtime.html#359538)

- Double click LVRTE2013std_downloader executable.  The LabVIEW Runtime Engine 2013 Downloader appears and begins downloading.

- When the pop-up dialog box appears click ok and you will be prompted to unzip the files to your desired location.  Use Browse to choose the desired location and leave the When done unzipping open: .|setup.exe box checked.

- Click okay to the dialog box confirming files have been successfully unzipped and if necessary allow the National Instruments Installer to make changes to your computer. This will launch the NI LabVIEW Run-Time Engine 2013 Installer.

- Follow the instructions to install LabVIEW Run-Time Engine 2013.

## To Connect Braille Unit with computer

USB-port of the Braille controller (home build) should be connected to USB-port of the computer. Then all the Braille pins should go down.

## Running the CombinatorialPlugFluidics Executable

1. After pressing Labview run button, select the right ‘Com Port’ and press ‘CONNECT BD’ then BD status should be on (LED lights up).

2. Once BD is connected, priming should be performed to check whether all the liquids are coming into the chip or not. This can be performed by clicking on green LED buttons that are labelled 1 to 32. Each number corresponds to the 2-pins in the Braille display (i.e. one pin moves up and the other pin, which is just above or below will go down).

3. Then moving to the next tab, where the user can set up combinatorial mixtures on-demand or using a pre-made configuration file in the format of .csv.

4. In manual mode, user selects specific sample or samples and then assigns valve opening time in seconds (using ‘Time (s)’ option). ‘The number of Replicates’ option is to produce that particular sample for defined number of times.

5. Once Data is selected, click on ‘ADD data’ button such that the newly added data can be seen in big Data matrix.

6. ‘SAVE Data’ button is used to save all the information in Data matrix and the user has to give file name.

7. Previously saved file can be uploaded into the Data matrix using ‘LOAD Data’ button. Once clicked, the user has to select a file from the computer.

8. The configurations can also be made in MS-Excel and save them as .csv. The .csv file can be uploaded by clicking on folder symbol in the ‘INPUT file’ section. Example csv file is shown below. Column ‘A’ denotes the valve opening time (s), Column ‘B’ denotes number of replicates Column ‘C’ denotes Sample number, and Column ‘D-E-F-G’ denotes which valves to be opened.

9. Moreover, if there is a mistake in the configuration, it can be deleted by pressing ‘DELETE Data’. This option will only useful to delete the latest configuration that was uploaded into the Data matrix.

10. The next button ‘RESET BD’ is used to reset braille display such that all the pins go down.

.

11. User can select which pins to be opened to generate barcodes. In this case, dye 1 for generating binary digit ‘1’ ( high flourescent signal), dye 2 for generating binary digit ‘0’ (low fluorescent signal), and End of barcode is defined as the mixture of dye 1 and dye 2.

12. Oil flush_Barcode is the number seconds waste configuration is set.

13. In Braille Application section, there are two tabs one for manual control and the other for Autosampler.

14. In the manual control mode, user gives the input for ‘Oil flush (s)’, which means all the aqueous samples go to waste. ‘Number of Cycles’ is to repeat the ‘n’ number of times of the configuarations that are in ‘Output Data matrix’

15. ‘Bracoding_Manual’ should be enabled to run the barcodes along with samples.

16. Interruptions could be done by pressing ‘OK’ button (Wait…) with specified ‘Delay Time’ (minutes). Once the delay time is elapsed, automatically the program starts from the next sample.

17. ‘STOP BD APP’ is used to immediately stop the program after finishing the current sample.

18. ‘STOP’ button to completely abort the program.

19. In the Autosampler mode, ‘Delay (s)’ is used to specify the number of seconds to wait after the auto sampler sends signal as ‘AS relay’.  ‘Auto Waste’ is similar to the ‘Oil flush’ in manual control. All the valves set to waste configuration. Once the Barcoding_AS is activated, then all the samples will be barcoded.

20. ‘STOP AUTOSAMP’ is used to immediately stop the program after finishing the current sample.