Study Protocol

Version 1

Assessment of hamstring muscle architecture using a large field of view ultrasound technique: an intrarater reliability study.

Authors: Kevin Cronin1, Shane Foley1, Giuseppe De Vito2,3, Sean Cournane⁴, Eamonn Delahunt2,3

¹ Radiography and Diagnostic Imaging, School of Medicine, University College Dublin, Dublin, Ireland.

2 School of Public Health, Physiotherapy and Sports Science, University College Dublin, Ireland.

3 Institute for Sport and Health, University College Dublin, Dublin, Ireland.

⁴ School of Physics, Sciences, University College Dublin, Ireland.

Kevin Cronin is an Assistant Professor in the School of Medicine, University College Dublin, Dublin, Ireland. He is registered as a PhD student in the School of Medicine, University College Dublin, Dublin, Ireland.

Shane Foley is an Assistant Professor in the School of Medicine, University College Dublin, Dublin, Ireland.

Giuseppe De Vito is a Full Professor in the School of Public Health, Physiotherapy and Sports Science, University College Dublin, Ireland.

Sean Cournane is an Assistant Professor in the School of Physics, University College Dublin, Dublin, Ireland

Conor McCarthy is a Consultant Rheumatologist in the Mater Misericordiae University Hospital, Dublin, Ireland.

Eamonn Delahunt is a Professor in the School of Public Health, Physiotherapy and Sports Science, University College Dublin, Ireland.

Title

Hamstring muscle architecture assessed sonographically using wide field of view: a reliability study

Abstract:

Hamstring injuries are very common in field sports. Muscle architecture has been suggested as a risk factor for hamstring strain injury. Various medical imaging techniques (Magnetic Resonance Imaging and Ultrasound) have been developed to assess muscle architecture. Ultrasound is often used to assess in vivo hamstring muscle architecture. The architecture of the hamstring muscles often extends outside the ultrasounds field of view. Previous ultrasound techniques used often generate questionable results. This study aims to describe the reliability of large field of view ultrasound and test – retest minimum detectable difference of the hamstring muscles.

Key words: Diagnostic imaging (MeSH); Ultrasonography (MeSH); Muscles, Skeletal; Hamstring muscles.

Introduction

The hamstring muscles are located in the posterior thigh. They are a complex muscle group consisting of three muscles: semimembranosus (SM), semitendinosus (ST) and biceps femoris (BF). The latter muscle has both a long and short head. All three muscle arise from the ischial tuberosity, which is a large osseous tubercle located at the superior pubic ramus of the ischium bone at the pelvis (Becciolini, Bonacchi and Bianchi, 2019). The ST and the BFlh have a common origin on the posteromedial aspect of the ischial tuberosity (Stepien et al., 2019; Philippon et al., 2015; Sato et al., 2012; van der Made et al., 2015). Some authors (Battermann et al., 2011; Miller, Gill and Webb, 2007) interpret the common origin of the ST and BFlh to be the medial aspect of the ischial tuberosity while (Feucht et al., 2015; Neuschwander, Benke and Gerhardt, 2015) claim the common origin to be located on the lateral aspect of the ischial tuberosity. The SM originates on the anterolateral aspect of the ischial tuberosity with its fibres twisting before forming a proper tendon (Stepien et al., 2019). All hamstring muscles have different insertion/attachment sites. In the proximal section of the hamstring muscle complex two main bursa can be identified (Stepien et al., 2019). However, these bursae are difficult to locate on ultrasound and to our knowledge no author has reported this sonographically. Based on anatomical dissection the more proximal bursa overlies the ischial tuberosity (Dierckman and Guanche, 2012; Guanche, 2015; Martin, 1968). The distal bursa is found next to the proximal BFlh muscle, precisely between the common attachment of the BFlh/ST common tendon and the SM proximal tendon adjacent to their bony attachment (Bejui et al., 1984).

The ST muscle is a superficial muscle that lies in the posteromedial area of the thigh. Proximally from its origin on the ischial tuberosity it runs distally overlying the semimembranosus muscle where it inserts onto the medial condyle of the tibia forming part of the pes anserinus (Stepien et al., 2019). The muscle belly of the ST is fusiform in appearance and is deeper in diameter than it is wide (Woodley and Mercer, 2005). Muscle fascicles run vertically down the belly of the muscle in a parallel fashion. The ST muscle is unique in the hamstring muscle complex in that it contains a mid-muscle belly tendinous inscription. This tendinous inscription is often referred to as “median raphe” or “muscle veil” and represents an inverted “V” on exploration (Kellis et al., 2012b; Kellis et al., 2010; Lee et al., 1988; van der Made et al., 2015; Woodley and Mercer, 2005).  Hamstring Strain Injury (HSI) is infrequent in the ST muscle (Askling et al., 2007; Askling et al., 2008; Cohen et al., 2011; Connell et al., 2004; De Smet et al., 2012; Hallén et al., 2014; Koulouris and Connell, 2003; Slavotinek, Verrall and Fon, 2002). Some authors have hypothesised that the presence of this tendinous inscription within the ST muscle belly acts as a protective property against HSI (Kumazaki, Ehara and Sakai, 2012; van der Made et al., 2015)

The SM muscle also lies posterior- medially and is quite superficial. From its origin proximally it descends over the medial condyle of the proximal tibia to the pes anserinus and descends under the ST (Stepien et al., 2019; Bejui et al., 1984). Muscle fascicles at the mid belly of the muscle have a connection to both proximal and distal tendons (van der Made et al., 2015). The muscle architecture distribution throughout the SM is heterogenous in appearance. Proximally the fascicles originate from the proximal tendon which originates from the ischial tuberosity. These fascicles orientate themselves distally where they attach to the lateral surface of the distal tendon (Woodley and Mercer, 2005). At the mid muscle section of the SM muscle the fascicles are orientated differently. Here the fascicles arise from the thin medial surface of the proximal tendon and travel inferiorly and posteriorly to insert into the lateral aspect of the distal tendon (Woodley and Mercer, 2005). The architecture arrangement in the distal segment of the SM is heterogenous in nature and difficult to observe the trajectory of a true fascicle. The fascicles here are bipennate in arrangement (Woodley and Mercer, 2005). The heterogenous arrangement of the muscle fascicles contribute to the bulky nature of the muscle distally.

The biceps femoris is comprised of two heads: the long (BFlh) and the short head (BFsh). The BFlh is a hamstring muscle prone to injury especially to athletes competing that involves high speed running. The proximal tendon of the BFlh forms a conjoint tendon with the ST muscle. The BFlh is a bipennate muscle (Wickiewicz et al., 1983; Woodley and Mercer, 2005; Klein Horsman et al., 2006; Ward et al., 2009). The BFlh contains a non-uniform muscle architecture and a distinct mid muscle aponeurosis (Freitas et al., 2018). It is a biarticular muscle that crosses both knee and hip joints, similar to the SM and ST muscles (Freitas et al., 2018). Distally the BFlh displays shorter fascicles with increased pennation fascicles than those seen proximally within the muscle which exhibit longer fascicle lengths but less pennate fascicles (Tosovic et al., 2016). The BFlh fuses with the BFsh at the distal thigh laterally forming an aponeurotic structure (Schache et al., 2012). Continuing distally this forms a conjoint tendon that attaches to the head of the fibula (Kellis et al., 2012a; van der Made et al., 2015; Klein Horsman et al., 2006) (Wangensteen et al., 2016). The BFsh muscle has much longer muscle fascicles, similar in length to the ST muscle but longer in length than the BFlh muscle (Woodley and Mercer, 2005). The muscle architecture of one hamstring muscle does not represent the muscle architecture of all remaining hamstring muscles (Kellis et al., 2012a).

The sciatic nerve innervates all hamstring muscles (Stępień et al., 2019). The sciatic nerve (also known as the ischiatic nerve) is the longest and largest single nerve in the body that starts in the lower back and runs down the lower limb via the buttocks (Ripani et al., 2006).  The ST, SM and BFlh are innervated by the tibial division of the sciatic nerve. The BFsh is innervated by the fibular division of the sciatic nerve. Both heads of the Biceps Femoris muscle and the SM are supplied by one motor branch whereas the ST muscle is supplied by two branches of the sciatic nerve. The sciatic nerve enters the lower limb by exiting the pelvis via the greater sciatic notch underneath the overlying piriformis muscle (Rha et al., 2016). It travels distally towards the popliteal fossa, dividing into its two branches approximately in the lower third of the posterior thigh (Adibatti and Sangeetha, 2014; An et al., 2010; Rab et al., 1997; Rha et al., 2016; Seidel et al., 1996; Woodley and Mercer, 2005).

Hamstring Strain Injury (HSI) is a negative disruption to the architecture of the BFlh, BFsh, ST and SM muscles (Bourne et al., 2017). A HSI most often occurs in a single hamstring muscle, rarely occurring in two or all three muscles at one time (Heiderscheit et al., 2010). HSI are frustrating for all parties involved; for coaches and staff who find themselves without their athlete for selection, for medical staff due to the difficult nature to diagnose and treat effectively but particularly for the athlete as inactivity bears financial difficulties and selection obstacles in team based sports (Hickey et al., 2014). They tend to reoccur and a return to play is capricious (Ernlund and Vieira, 2017; Carlson, 2008). HSI’s are most common in sport and are frequent in field sports but often present in other sports also (Askling et al., 2008). Soccer – the most popular sport in the world, attributes 37 percent of all muscular injuries to HSI’s (Askling et al., 2013; van der Horst et al., 2015). HSI’s are the most frequent injury in soccer (Askling et al., 2013; van der Horst et al., 2015). Incidence and trends are changing relative to HSI’s. There is a mean increase of HSI’s of 4 percent per year reported; with the rate of injuries occurring in training sessions increased more than those occurring in competitive activities (Lempainen et al., 2015; Ekstrand et al., 2016). Hamstring injuries are typically classified into three main categories; grade I (mild), grade II (moderate) or III (severe) (Mason, Dickens and Vail, 2007; Schneider-Kolsky et al., 2006). Depending on the grade of strain and the type of sport involved, return to play variations exist. Where surgery is not needed runners can take as long as 16 weeks, on average, to return to sport without restrictions, whereas dancers can take as long as 50 weeks to return to full activity (Ernlund and Vieira, 2017). Furthermore in professional soccer the average time an athlete is absent from competition due to an HSI is typically 14 days (Ernlund and Vieira, 2017). HSI’s are the primary cause for athlete absence from competitive action (Carlson, 2008; Lempainen et al., 2015; Kerkhoffs et al., 2013; van Dyk et al., 2016). Hamstring injuries have been consistently identified as one of the most common injuries sustained by professional rugby union players (Fuller et al, 2013; Fuller et al, 2016; England Professional Rugby Injury Surveillance Project). They have a high injury burden (i.e., total days injured and unavailable for selection) and a high recurrence rate.

High reinjury rates after an acute hamstring injury are common (Wangensteen et al., 2016). Reinjury rates are reported to range from 14 percent to 63 percent within the same competitive season (Visser et al., 2012; Vos et al., 2014; Ekstrand et al., 2011; Ekstrand et al., 2012; Hallén et al., 2014; Silder et al., 2013). It is apparent that HSI’s appear to be on an upward curve despite the best efforts of coaches and medical personnel to reduce their incidence. (Silder et al., 2013) noted that HSI’s occur at the exact same location as the initial injury site while (Koulouris et al., 2007) noted that the reinjury site was the musculotendinous junction located within the muscle tendon unit. A recurrent HSI is often more severe than the initial HSI (John et al., 2006; Ekstrand et al., 2011). In elite rugby union in Australia recurrent HSI’s have traditionally always been a dilemma with recurrent HSI’s rates as high as 34 percent per season (Seward et al., 1993). However more recent data in elite ruby union conveys approximately 22 percent off all HSI’s were recurrent HSI’s (John et al., 2006). The high recurrence rates perhaps suggest that current rehabilitation practices are not fully addressing the underlying risks factors associated with recurrent HSI’s (Bourne et al., 2017).

The architectural characteristics of a muscle refer to the physical arrangements of muscle fascicles and determine a muscles mechanical function (Salimin et al., 2018). The trajectory of the muscle fascicle between both the superficial and deep aponeurosis is referred to as the pennation angle (Kawakami et al., 2000; Baechle et al., 2008). Greater pennation angles allow a greater amount of muscle fascicles to attach to an aponeurosis thus increasing its physiological cross-sectional area of the muscle (Albracht, Arampatzis and Baltzopoulos, 2008; Bamman et al., 2000; Blazevich et al., 2009). In vivo ultrasound assessment of the architectural characteristics of the hamstring muscle measure the pennation angle by observing where the muscle fascicle inserts into the deep aponeurosis of the muscle (Becciolini, Bonacchi and Bianchi, 2019; Freitas et al., 2018a; Pimenta, Blazevich and Freitas, 2018; Kellis et al., 2010; Kellis et al., 2012b; Franchi et al., 2014; Brennan et al., 2017). An increase in pennation angle will increase the cross-sectional area of the muscle, thus increasing the number of muscle fascicles within the muscle (Salimin et al., 2018). Greater fascicle lengths represent longer and more sarcomeres in series (Blazevich and Sharp, 2006; Earp et al., 2014; Albracht et al., 2008).Architectural characteristics of muscle influence the maximal force output, muscle length, contraction velocity and susceptibility to injury (Brockett, Morgan and Proske, 2004). Muscle architecture is adaptable and responds to a range of stimuli (Bourne et al., 2017).  Longer muscle fascicles exhibit less shortening over a given amount of total muscle shortening than shorter fascicles would (Blazevich et al., 2009; Blazevich and Sharp, 2006) (Kumagai et al., 2000) (Abe, Kumagai and Brechue, 2000). According to this hypothesis greater fascicle lengths are beneficial for force production at fast movements and reduces the athlete’s susceptibility to HSI.

Often when an athlete suffers an initial HSI the immediate pain halts their progress in the game or competition (Ernlund and Vieira, 2017). When the muscle is functioning beyond its natural boundaries microscopically the muscle fibers become damaged gradually leading to a tear. Retrospective reports have identified numerous factors of maladaptation’s associated with previous HSI that contribute to reinjury; deficits in the rate of torque development (Opar et al., 2013) and an reduction of BFlh muscle volume (Silder et al., 2008). A reduction in fascicle lengths exhibit weaker muscles (Brockett et al., 2004; Timmins et al., 2015). It is understood that there is a reduction in fascicle lengths post HSI (Brockett et al., 2004; Fyfe et al., 2013). Timmins et al. (2005) explains that this is “troublesome” for reinjury because shorter fascicles would increase the muscles susceptibility to eccentrically induced microscopic damage, which could most likely be a precursor to macroscopic damage in the form of strain injury. However, it is still not understood if a previously strained hamstring muscle contains shorter fascicles prior to the injury happening. Evidence suggests that the BFlh muscle displays shorter fascicles in individuals with a history of strain in injury (Timmins et al., 2015). Possessing shorter fascicles has been suggested to increase the likelihood of microscopic muscle damage as a consequence of repetitive eccentric training such as running (Fyfe et al., 2013). Thus, prospective studies will hope to determine if shorter muscle fascicles increase the risk of future injury in human hamstring muscles.

Over the past thirty years various medical imaging techniques have been used to assess fascicle geometric disposition within skeletal muscle. Up to the early 1990’s this was studied in cadaver specimens (Huijing et al., 1985;Wickiewicz et al., 1983). However, with cadaveric specimens bears many limitations primarily difficulties associated with sample size, due to the limited availability of donor tissues (Kellis et al., 2009). Furthermore, all these studies examined muscle architecture in specimens from elderly individuals (Kwah et al., 2013). Tissue from cadaveric specimens is generally sarcopenic tissue and as such the muscle architecture would vary from the athletic population where the majority of individuals are ages between 18-35 years of age (Blazevich and Sharp, 2006; Opar, Williams and Shield, 2012).

To overcome these limitations medical techniques have been applied to assess in vivo architectural characteristics of skeletal muscles. Brightness mode (B-mode) ultrasound imaging is deemed most popular, because it is relatively inexpensive, it is well tolerated by the athlete and yields accurate results with improving techniques and advancing imaging applications (Becciolini et al., 2019; Bolsterlee, Gandevia and Herbert, 2016b; Bourne et al., 2017; Brennan et al., 2017; Connell et al., 2004; e Lima, da Matta and de Oliveira, 2012; Franchi et al., 2014; Franchi et al., 2018; Gabison et al., 2018; Adibatti and Sangeetha, 2014; Kellis et al., 2012a; Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009; Kumagai et al., 2000; Kwah et al., 2013; Legerlotz, Smith and Hing, 2010; Narici et al., 2003; Rab et al., 1997; Timmins et al., 2015; Wickiewicz et al., 1983; Woodley and Mercer, 2005).

Skeletal muscle has a high-water content therefore on ultrasound imaging will appear mostly hypoechoic (i.e., dark) (Ihnatsenka and Boezaart, 2010). Perimysial connective tissue gives muscle a speckled appearance and will appear hyperechoic (white) within the muscle belly (Franchi et al., 2018). Ultrasound can image skeletal muscle in two planes: transverse and longitudinally. It is in the longitudinal plane we can extract most information about a muscle’s architectural characteristics (fascicle length, muscle thickness and pennation angle).

Different muscles have distinct structural arrangements (Friederich and Brand 1990; (Wickiewicz et al., 1983). The human gastrocnemius muscle, frequently assessed with ultrasound (Mathevon et al., 2015; Hauraix et al., 2015; Bolsterlee et al., 2016b; Bolsterlee, Gandevia and Herbert, 2016a; Bolsterlee et al., 2015; Rosenberg et al., 2014) and  vastus lateralis (Varanoske et al., 2017; Ticinesi et al., 2018; Hauraix et al., 2017; Khoshkhoo et al., 2016; Raj, Bird and Shield, 2012; Arroyo et al., 2018) present with muscle fascicle lengths ranging from 3cm to 5cm for the gastrocnemius and 6cm to 8cm for the vastus lateralis muscle. Additionally, the human sartorius muscle displays muscle fascicle lengths that exceed 20 cm (Heron and Richmond, 1993).

Ultrasound assessment of the hamstring muscle group is often favorable for ultrasound assessment and analysis as it is quite a superficial muscle and is easily penetrated with the ultrasound beam. The BFlh muscle is the most common hamstring muscle assessed via ultrasound (Kwah et al., 2013; Potier, Alexander and Seynnes, 2009; Bourne et al., 2017; Timmins et al., 2014; Timmins et al., 2016b; Timmins et al., 2016a; e Lima et al., 2012; Tosovic et al., 2016; Kellis et al., 2012a; Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009). Few studies (Kellis et al., 2010) have assessed the architecture of the semitendinosus muscle via ultrasound and to the authors knowledge no studies (in vivo) have assessed the architectural characteristics of the semimembranosus muscles. Perhaps some reasons for this include; they are infrequently injured in comparison to the BFlh muscle (Bourne et al., 2017; Askling et al., 2007; Ekstrand et al., 2011; Ekstrand et al., 2016; Hägglund et al., 2016; Ernlund and Vieira, 2017) therefore may not be a great clinical concern. Additionally, the architecture is heterogenous in nature, and may prove difficult to track architectural characteristics on ultrasound (Balius et al., 2019).

There are many inconsistencies in technique and analysis across studies to assess the BFlh muscle to pursue a similar pathway that is valid, reliable, reproducible, repeatable and transparent (Potier, Alexander and Seynnes, 2009; Bourne et al., 2017; Timmins et al., 2014; Timmins et al., 2016b; Timmins et al., 2016a; e Lima et al., 2012; Tosovic et al., 2016; Kellis et al., 2012a; Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009). Various studies (Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009; Bourne et al., 2017; Timmins et al., 2016b; Timmins et al., 2015; Freitas et al., 2018a; Pimenta et al., 2018) have used different methods to standardise the transducer orientation and location, however no general consensus is established regarding the best process to limit measurement error. Futhermore these studies lack detail on optimal technique to capture reproducible images and often fail to consider and mention how they tracked the same fascicle over time. However a recent systematic review (Kwah et al., 2013) reported the reliability and validity of B-mode ultrasound in measuring fascicle length and pennation angle across many muscles. This review informs us that B-mode ultrasound is a reliable and valid in vivo method to quantify muscle architecture in comparison to cadaveric samples. The study conducted by (Freitas et al., 2018a) overcomes some limitations (same data acquisition site) associated with previous studies (Kellis et al., 2010; Timmins et al., 2015), identifying a reliable method (ICC >0.9) to assess midmuscle architecture of the BFlh when the same rater performs the image acquisition and digitisation process. This study identified the myotendinosus junction (proximal and distal) of the muscle in question (BFlh) and a mark was drawn on the skin. The distance between both marks was measured and accepted as the BFlh muscle length. This is in contrast to other studies (Timmins et al., 2014; Timmins et al., 2016b; Timmins et al., 2016a; Timmins et al., 2015; Kellis et al., 2012a; Kellis et al., 2012b; Kellis et al., 2009; Kellis et al., 2010; Potier et al., 2009) who use bony landmarks to calculate muscle length and decide on a same data acquisition site. This technique is difficult to reproduce as bony landmarks are not constant if the athlete is not set up in an identical position as the previous time. To identify the specific region within the BFlh muscle (Freitas et al., 2018b) scanned the muscle belly in transverse and longitudinal planes until the resultant image cleary identified both the superficial and deep aponeurosis in a parallel orientation and where a muscle fascicle was clearly identified. The ultrasound transducer location on the BFlh muscle was noted and the distance from the distal myotendinosus junction was recorded. Identifying this location again differs from previous studies (Kellis et al., 2012a; Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009; Potier et al., 2009; Timmins et al., 2014; Timmins et al., 2016b; Timmins et al., 2016a).  Furthermore, it is necessary to clearly identify the exact location site to acquire muscle architecture data as the architecture within a muscle can vary considerably (Timmins et al., 2015). For example the BFlh posesses proximal fascicle which are longer in length compared to distal fascicles (Bennett, Rider, Domire, DeVita, & Kulas, 2014). Therefore standardising the assessment location is paramount to minimise any variations due to inconsistencies in the scanning site (Timmins et al., 2015). Clearly this (Freitas et al., 2018b) is the current “gold standard” reproducible method to identify the same location site with a proven accurate technique, yet bears associated limitations like similar studies in the small field of view to capture the image and in the image analysis section which assumed the fascicle travel in linear fashion.

One key limitation of ultrasound imaging when assessing the muscle architecture of the hamstring muscles is its relatively small field of view. The field of view is determined by the size of the linear array transducer (Ihnatsenka and Boezaart, 2010). Commercial linear array transducers suitable for assessing the muscle architecture operate with field of views typically between four to six cm (Franchi et al., 2018). These fields of view are typically shorter than the fascicles under investigation (Blazevich, Gill and Zhou, 2006). In this instance fascicle length is estimated with various linear approximations using measured muscle thickness and pennation angle values (Blazevich et al., 2006; Kellis et al., 2012b; Kellis et al., 2009; Kellis et al., 2010; Timmins et al., 2014; Timmins et al., 2016a; Timmins et al., 2015). These methods fail to consider the variability associated with fascicular curvature and as such are prone to error (Darby, Li, Costen, Loram, & Hodson-Tole, 2013; Rana, Hamarneh, & Wakeling, 2014).

Extended Field of View (EFOV) was introduced by many researchers to overcome this key small field of view limitation (Adkins, Franks and Murray, 2017; Noorkoiv et al., 2010; Seymore et al., 2017; Simpson et al., 2017). This technique impliments texture mapping algorithms to merge sequences of images collected during dynamic scanning to reconstruct large composite images (Franchi et al., 2018). It requires a high skill from the operator however (Noorkoiv et al., 2010) reports high levels of reliability with an ICC of 0.99. It is important to note that EFOV is not suitable for all muscles particulary the hamstring muscles as the architecture distributuion within these muscles is hetergenous in nature (Blazevich and Sharp, 2006). Thefore this is a challenge practically, as it is cumbersome and extremly difficult to satisfy the fundamental requirement of keeping the transducer parallel to the fascicle plane when covering a large area of interest (Franchi et al., 2018). Additionally a limitation of this technique is observed in the use of a linear array transducer over curved surfaces such as the hamstring muscle in the poster thigh – where exerting any pressure on the muscle to ensure proper fascicle alignment mighy depress the underlying muscle and influence measurement results (Franchi et al., 2018). This technique requires extensive operator training and still will not guarantee high reproducible/repeatable values. Not all ultrasound applications support EFOV and this technique is not popular in assessing the architecture of muscles during contraction, which is also quite popular (Franchi et al., 2018).

Furthermore to obtain a field of view without estimating the remaining fascicle length (extrapolation method with a small field of view) or utilising EFOV some authors (Brennan et al., 2017; Herbert et al., 2011) have suggested using a dual transducer system. This system places two 5cm linear array transducers side by side and held together in a small frame. According to (Brennan et al., 2017) this new technique allows more precise estimations of muscle fascicles. However this technique means that there is a 2cm gap between both images taken and could be tedious and difficult to “stitch” images together to calculate muscle architecture properties.

One way to overcome all these difficulties to assess muscle architecture in muscles with long fascicles is to use an ultrasound transducer with a long field of view. Such a transducer would capture the full fascicle in its length and not rely on estimates via linear exploration equations to calculate the correct fascicle length. Therefore by implementing a correct technique, the full fascicle can be identified on the sonogram and be saved for later analysis. A large field of view transducer like this would capture static images unlike EFOV, which requires dynamic scanning, and allows a transparent, valid, repeatible and reproducible technique to persue. Temporal resolution is often limited with transducers with larger field of views however with advancements in technology this issue is minimal to none, and (this transducer) still acquire higher quality images than other studies  ('<ECR2014 Ultrasound Anatomy.pdf>,' ; Abe et al., 2000)which used now dated ultrasound applications.

Acquired ultrasound images are often manually analysed identifying muscles fascicles and pennation angles (Abe et al., 2000; Timmins et al., 2014; Timmins et al., 2016b; Timmins et al., 2015; Cronin et al., 2011; Cronin and Lichtwark, 2012). Muscle fascicle length was estimated by placing a minumum of two points that represented the proximal and distal ends of the fascicles inserting into the deep and superficial aponeurosis (Cronin and Lichtwark, 2012). This method is incorrect as we know fascicles do not travel in a straight line, are curved in nature, and the fact that the muscle belly is not homogenous in thickness throughout (Blazevich and Sharp, 2006). Some studies (Af Klint et al., 2010; Cronin et al., 2009) used a three point tracking procedure along the fascicle (proximal, mid and distal fascicle) in an attempt to account for fascicle curvature. This process was either performed by a number of raters or a single rater and was performed a number of times. This procedure was tedious, time consuming and depending on the number of raters was subjective and prone to error (Drazan, Hullfish and Baxter, 2019). To calculate the pennation angle researchers calculate the angle the fascicle inserts into the deep aponeurosis. Many studies track the architectural characterstics of muscles over time in eith a passive or active state. This produces huge amounts of data. In the past decade many authors experimented with automated analysis to track architectural characterstics of skeletal muscles. Several different methods have been developed with the view of efficient and effective ultrasound data analysis of skeletal muscle (Loram, Maganaris and Lakie, 2004; Loram, Maganaris and Lakie, 2006; Herbert et al., 2011; Cronin et al., 2011; Cronin et al., 2009; Cronin and Lichtwark, 2012; Zhao and Zhang, 2011; Miyoshi et al., 2008; Rana, Hamarneh and Wakeling, 2009). Not all these software applications are free access however we experimented with some of these automated analysis that were available to us. None fulfilled our requirements for various reasons; the software did not correctly identify the muscle, and/or more often than not we were unable to analyse the same fascicle on a particular sonogram. Due to the relative novelty of some of these methods, and the specific experimantal conditions under which each of these has been tested it was not possible to identify an optimal method of tracking suitable to our study. What we propose is to use a semi automated image analysis using the Matlab software. We have developed our own algorithm within this application that will encorporate a six point tracking procedure (two points  at the proximal, mid and distal fascicle). This will account for the curvature illustrated along the full fascicle captured. A measurement in millimeters will be given. This is termed the numerical method. The full fascicle length is then extracted and placed within a interpolated curve. The numerical method interpolates between each of the selected points, using a cubic spline interpolation. Then the curve is differentiated along the interpolated curve. The trapezoidal rule is then used to sum the length.The percentage difference between the numerically calculated (using points on the digitised sonogram) and the mathematically determined (true) is calculated. The difference between both sets of measurements should be minimal. We tested this semi automated analysis for measuring fascicle length. We found the difference in millimeters between the numerical method (6 point system) and the interpolation method (extracted fascicle onto a interpolated curve) to be a 0.2 percent difference.

Aims

The aim of our study is to determine the intrarater reliability of a wide field of view ultrasound technique to assess the architectural characteristics (thickness; fascicle length; fascicle pennation angle) of the hamstring muscles.

Hypothesis

We expect to achieve excellent intrarater reliability (ICC values >0.90) when using the large field of view ultrasound technique to assess the architectural characteristics of the hamstring muscles.

Design

This study is designed as an intrarater reliability study.

Technique to calculate muscle length

An initial scan of the Hamstring Muscle Group should be completed. To calculate the length of each muscle; Locate the proximal myotendinous junction by orientating the transducer transversely along the long axis of the muscle. When the smallest proximal muscle section is observed, the transducer is orientated longitudinally at the intersection of the deep and superficial aponeurosis. Mark on the skin “X” with a washable Crayola skin marker. This “X” represents the proximal MTJ. Next locate the distal MTJ in similar fashion – mark on the skin “X” with a washable Crayola skin marker. The distance between the two “X” marks represents the muscle length. Before a mark (X) is made – confirmation of both proximal MTJ and distal MTJ should be assessed three times. This ensures an accurate marking of the MTJ occurs and thus an accurate measurement of the entire muscle. This technique is identical to the technique implemented by Freitas et al (2017) when assessing the BFlh muscle architecture which yielded high intra rater ICC (0.95).

Once both proximal MTJ and distal MTJ are marked; the distance between both “X’s” represents the true length of the muscle. The length is measured with tape in millimetres.

Standardisation of specific region

Each muscle is divided into a proximal and distal zone. The proximal zone of the muscle represents the upper half of the muscle length while the distal zone represents the lower half of the muscle length. The specific region for examination of muscle architecture (Fascicle length, pennation angle, muscle thickness) is chosen in each zone by scanning the zone in both transverse and longitudinal planes until the sonogram clearly captures the superficial and deep aponeuroses in parallel orientation, where the fascicles are easily identifiable and attaching to the deep aponeurosis of the muscle. This is somewhat similar to the assessment of BFlh architecture by Freitas et al. (2017) yet differs in that they only acquired data from the mid muscle region of the BFlh. Other studies failed to convey their specific region for data acquisition.

Standardisation of region on Ultrasound Transducer

The Ultrasound transducer has a Field of View (FOV) of 92mm. Once the proximal/distal zone is scanned initially to identify a suitable fascicle to image, the transducer will be orientated longitudinally. A mark “X” will be made at the proximal end of the transducer. This mark will then be measured with tape in millimetres, its distance from the proximal MTJ. Marking an “X” at the proximal end of the transducer utilises the full 92mm field of view. This will ensure that the full fascicle is captured on the sonogram. Additionally, the fascicle appearing at the bottom left of the screen (inserting into the deep aponeurosis) ensures that this is the same fascicle being monitored over time.

Proximal Zone

Figure 1: Each muscle will be divided into a proximal and distal zone. The proximal end of the transducer will be placed at a fascicle clearly visible. This location will be marked and recorded from the proximal MTJ to ensure the same fascicle is tracked over time.

Data Acquisition sites

Please also refer to the flowchart seen in figure 2.

Bicep Femoris Long Head: At the proximal and Distal Zones, Fascicle Length, Pennation Angle and Muscle thickness will be recorded three times.

Bicep Femoris Short Head: Only the Muscle thickness will be recorded three times. The BFsh contains fascicles that extended beyond the transducers FOV.

Semitendinosus: Only the Muscle thickness will be recorded three times. The ST contains fascicles that extended beyond the transducers FOV.

Semimembranosus: At the proximal Zone only will fascicle length, pennation angle and muscle thickness be recorded. The distal zone of the semimembranosus muscle displays heterogenous architecture difficult to quantify.

Figure 2: A flowchart illustration proposed data acquisition sites of the hamstring muscle group.

Image Quality

Quality Assurance

The benefits of implementing regular quality control tests are widely recognised in terms of image quality (Sipila, Mannila and Vartiainen, 2010). Quality control tests may be tedious to perform but are worthwhile because it may be difficult or even impossible to detect calibration and measurement distortion from examination of images alone (Sanders and Winters, 2006).  A series of tests (listed below) will be carried out on a multi-purpose, multi-tissue ultrasound phantom (CIRS phantom, certified to ISO 9001:2008 standards). Tests will be carried out at least two times per year, for the duration of the project. Baselines will be established from a series of tests conducted. Allowable deviations from baselines measurements will be established. If the baseline is exceeded (established cut off point), the operator will run the tests again and where baselines again exceed the cut-off point, the equipment will be removed from practice and the equipment manufacturer will be contacted to rectify the issue.

Quality Control Tests

Uniformity Testing

Uniformity is defined as the ability of the ultrasound application system to display echoes of the same magnitude and depth with equal brightness on the display monitor. This is a good test to ensure all crystals within the transducer are functioning.

Depth of Penetration

This is the greatest distance in a phantom for which echo signals caused by scattering in the background material can be detected on the display. Again, this is a good test to ensure all crystals within the transducer are functioning.

Axial and Lateral Resolution Testing

Axial resolution is the ability of an ultrasound system to resolve objects in close proximity along the axis of the beam. It determines how close two objects can be along the axis of the beam can be detected as two distinct objects. Lateral resolution, similar to axial resolution, however this determines the resolution perpendicular to the beam axis.

Ultrasound examination of the hamstring muscles requires a demanding technique from the operator and an in-depth anatomical knowledge of the posterior thigh (Balius et al., 2019). The skill of the operator is of utmost importance when assessing the complex architecture of the hamstring muscle complex (Timmins et al., 2015). In all instances an initial scan of the ST, SM, BFlh and BFsh will ensue as per (Balius et al., 2019) identifying landmarks and common structures before any measurements will ensue. A single assessor aids in the reliability of measurements both at acquisition and at data analysis (Klimstra et al., 2007; Kwah et al., 2013).

B-mode

Brightness mode (B-mode) ultrasound relies on the transmission of echo pulses from an ultrasound transducer (Franchi et al., 2018). The ability of the ultrasound transducer to penetrate skeletal is proportional to its wavelength whereas the spatial resolution is inversely proportional to it (ibid et al., 2018). Skeletal muscle is superficial therefore is easily penetrated and requires only a high frequency wavelength (Zagzebski, 1996). Frequencies of between 5-10MHz are often used to analyse skeletal muscle (Abe, Kumagai and Brechue, 2000; Aeles et al., 2017; Arroyo et al., 2018; Balius et al., 2019; Becciolini, Bonacchi and Bianchi, 2019; Blazevich et al., 2009; Blazevich, Gill and Zhou, 2006; Bolsterlee, Gandevia and Herbert, 2016b; Bourne et al., 2017; Brennan et al., 2017; Cho and Kim, 2016; Cronin et al., 2011; e Lima, da Matta and de Oliveira, 2012; Farris and Lichtwark, 2016; Franchi et al., 2014; Freitas et al., 2018; Hauraix et al., 2017; Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009; Klimstra et al., 2007; Koulouris and Connell, 2003; Kumagai et al., 2000; Kwah et al., 2013; Mairet, Maïsetti and Portero, 2006; Mathevon et al., 2015; Miyoshi et al., 2008; Narici, 1999; Pimenta, Blazevich and Freitas, 2018; Rab et al., 1997; Raj, Bird and Shield, 2012; Rana, Hamarneh and Wakeling, 2009; Seymore et al., 2017; Simpson et al., 2017; Ticinesi et al., 2018; Timmins et al., 2014; Tosovic et al., 2016; Zhao and Zhang, 2011).With reference to muscle architecture (Cronin and Lichtwark, 2012) explain that modern ultrasound devices possess the necessary resolution to optimally analyse ultrasound images efficiently.

Focus

The focus must be set at the region of interest (40mm). This is where the ultrasound beam is at its narrowest, and strongest, thus improves the resolution (lateral resolution) of the overall image(Zagzebski, 1996). Thus, it will add clarity to the image, cleansing and sharpening the sonogram enabling a vivid image (Ihnatsenka and Boezaart, 2010), which is important when viewing the fascicle after the digitisation process. A single broad focal zone will be implemented – that spans the entire muscle thickness in the longitudinal plane.

Gain

The Gain control adjusts the overall amplification of the returning reflections displayed on the ultrasound image.  The gain adjustment can be likened to the brightness adjustment on a television. There seems to be a vast difference towards gain preference. Some studies (Kellis et al., 2012a; Kellis et al., 2010; Kellis et al., 2012b; Kellis et al., 2009) often prefer an overall darker sonogram to track fascicles whereas more recent authors choose brighter sonograms to observe the muscle architecture (Adkins, Franks and Murray, 2017; Blazevich et al., 2006; Franchi et al., 2018; Freitas et al., 2018; Pimenta et al., 2018). A reason for this could be newer ultrasound technology possesses many applications to improve and enhance the resolution of the sonogram (Cronin and Lichtwark, 2012; Ihnatsenka and Boezaart, 2010). An optimal gain was set based on analysis of the literature by the primary author of images provided in all electronic and hard copy literature. Additionally, a “Hamstring parameter setting” was developed by an application specialist in the company of the primary researcher, a medical physicist and a sonographer with over ten years clinical experience. This selected gain will remain constant throughout the duration of the study.  Unit of measurement is in decibels (Db) (Zagzebski, 1996).

Depth

The depth simply determines “how far” into the body one wishes to image. Depth is in increments of centimetres (Zagzebski, 1996). Initially it was assumed the same depth will remain throughout each examination. However, it is wise to optimise your field of view to enable the full fascicle to be captured on the sonogram and analysed later. This will often require the operator to alter the depth parameter specific to muscle location and per athlete. At the analysis section once all images are digitised, the depth at each location will be recorded on an Excel spreadsheet and will be reviewed and referred to next time the athlete is scanned. This ensures similar parameters (in this case depth) is implemented each time. It is important to note that as body habitus differs between individuals, some hamstring muscles appear deeper on ultrasound than others. Thus, it would be incorrect to standardise a depth increment into this protocol. Depth will be set to 80mm.

Transducer orientation

The most accurate measurements are made when the transducer are orientated so that the image plane aligns with the muscle fascicles, and for measurements of pennation angle when the image plane intersects the aponeurosis (i.e. fascicle is seen inserting into the deep aponeurosis) (Bolsterlee, Gandevia and Herbert, 2016a). However, this can be practically difficult as the smallest mal manoeuvre of the transducer can distort the image. To limit error the primary researcher will implement recommendations on transducer orientation as per (Bolsterlee et al., 2016a)

- ensure the transducer is always perpendicular to the skin surface. If this is not implemented pennation angles will be overestimated.

- ensure the transducer is aligned to the image plane of the muscle fascicles. A tilt error of 5.5⁰ induces a >2mm error in fascicle length, tilt error of 10⁰ induces a >4mm error while a tilt error of 20⁰ induces an error of almost 7mm.

- ensure the fascicle in question is seen inserting into the deep aponeurosis of the muscle. The author acknowledges that this was conducted on a gastrocnemius muscle and might prove challenging in larger and deeper muscles such as the hamstring muscles.

Room Lightening

To replicate the clinical setting, light will be powered off. The researcher scans with the light off clinically.

Setting

A single room in the clubhouse of each volunteers. All windows will be blacked out to represent the clinical setting of the researcher who clinically scans with ultrasound in total darkness. All clubhouses are within Dublin, Ireland. Volunteers will not receive any payment for participation in this study.

Subjects

We will test 20 consecutive male volunteers. The volunteers were recruited through the dissemination of flyers (see appendix) throughout numerous sporting organisations clubhouses (amateur GAA teams) in Dublin.

Each volunteer will need an Ultrasound Examination Entry to be created on the ultrasound application system. Participant Examination Entry on the ultrasound application system should not include the participants name but should be coded according to the participant e.g. “ParHamRelX” (Par = Participant, Ham = Hamstring, Rel = Reliability, X = Participant Number).

Inclusion criteria

- Male gender

- Athletes

Exclusion criteria

- Age less than 18 or more than 65 years.

- Not in a good general health

Storage

The ultrasound application system used in scanning the volunteers (Hitatchi Noblus) is a portable ultrasound machine. It facilitates extraction of data from the machine via a USB port. All images will be saved on an USB key and uploaded and digitised for image analysis on the Image J software. All material stored/assessed on either the USB key or Image J will be password protected. The USB key will be stored in a locked filing cabinet in the primary researcher office in University College Dublin. This office is a locked shared office only accessible with a key.

Sample size considerations

Sample size estimation is an important initial step when researchers are planning the design and conduct of their study (Walter et al, 1998). The sample size for a test-retest reliability study depends on the minimally acceptance level of reliability ᵨₒ, the target level of reliability ᵨ (measured as interclass coefficient), and the number of repetitions of the measurement n. The sample size was calculated by use of G*Power software. Considering an effect size of 0.9, significance level of 0.05 and a statistical power (p) of 0.8, the minimal sample size was 16. In summary, 20 subjects participated in this study, p = 0.87.

Statistical Analysis

Reliability refers to the consistency of a test or a measurement (Weir, 2005). This study will assess the reliability of a new ultrasound measurement technique used to quantify muscle architecture (fascicle length, pennation angle and muscle thickness). The calculation of reliability starts with the performance of repeated measures ANOVA (test retest). Each volunteer will have both limbs assessed over two sessions. Each session will be approximately one week apart.

Intraclass correlation coefficient (ICC): it measures the relative homogeneity within sessions in relation to the total observer variation between sessions. For this analysis an absolute agreement will be selected, and ICC for grouped measurements (three measurements will be acquired per zone in each muscle for each muscle) (fascicle length, pennation angle and muscle thickness). An ICC value >0.9 indicates excellent reliability.

Relevance

To the knowledge of the authors, there has not been any study on the reliability of a large FOV (>6cm) linear array ultrasound assessment of the muscle architecture of the hamstring muscles to date. Most studies on quantifying muscle architecture of skeletal muscle used a transducer with a narrow field of view which was typically shorter than the muscle fascicles. Thus, muscle fascicle length was estimated with various linear approximations and failed to consider the variability associated with fascicular curvature and as such are prone to error.

Risks

No risks related to the ultrasound assessment of the hamstring muscles are known.

Financing and Insurance

The purchase of an ultrasound transducer (€6000) was supported by Professor Eamonn Delahunt and Professor Giuseppe De Vito. Additionally, the 1st author will have 75% of his PhD registration fees covered for this first three years of the overall project.

Ethics

Ethical exemption was granted by the university’s ethics committee, HREC Ref no: LS-E-19-83.

Authorship

Authorship for the project will be based on the Uniform Requirements for Manuscripts of the International Committee of Medical Journal Editors. According to these requirements, authorship credit should be based on 1) substantial contributions to conception and design, acquisition of data, or analysis and interpretation of data; 2) drafting the article or revising it critically for important intellectual content; and 3) final approval of the version to be published. Authors should meet conditions 1, 2, and 3. Changes in the tasks of the authors listed below may lead to loss of authorship for some contributors or addition of new authors during the course of the project.

Tasks of Authors

Study design: all the authors

Draft of the study protocol: Mr Kevin Cronin, Professor Eamonn Delahunt, Dr Shane Foley

Logistic organisations: Mr Kevin Cronin, Professor Eamonn Delahunt, Dr Shane Foley

Performance of the experiment: Mr Kevin Cronin

Analysis of data: Mr Kevin Cronin, Professor Eamonn Delahunt, Dr Shane Foley, Dr Sean Cournane

Interpretation of findings: all authors

Draft of article: Mr Kevin Cronin, Professor Eamonn Delahunt, Dr Shane Foley, Giuseppe de Vito, Dr Sean Cournane

Final version of article: all authors

Senior responsibility of all processes: Professor Eamonn Delahunt and Dr Shane Foley

The study will be part of the dissertation of Mr Kevin Cronin

Time plan

Volunteer recruitment: from April 2019 to June 2019

Ultrasound assessment of each volunteer: from September 2019 to October 2019

Data analysis: November 2019

First draft of paper: January 2020

Submission to peer – reviewed journal: February 2020.

References

Abe, T., Kumagai, K. and Brechue, W. F. (2000) 'Fascicle length of leg muscles is greater in sprinters than distance runners', Medicine and Science in Sports and Exercise, 32(6), pp. 1125-1129.

Adkins, A. N., Franks, P. W. and Murray, W. M. (2017) 'Demonstration of extended field-of-view ultrasound's potential to increase the pool of muscles for which in vivo fascicle length is measurable', Journal of Biomechanics, 63, pp. 179-185.

Adibatti, M. and Sangeetha, V. (2014) 'Study on variant anatomy of sciatic nerve', Journal of Clinical and Diagnostic Research, 8(8), pp. AC07-AC09.

Af Klint, R., et al. (2010) 'Afferent contribution to locomotor muscle activity during unconstrained overground human walking: An analysis of triceps surae muscle fascicles', Journal of Neurophysiology, 103(3), pp. 1262-1274.

Albracht, K., Arampatzis, A. and Baltzopoulos, V. (2008) 'Assessment of muscle volume and physiological cross-sectional area of the human triceps surae muscle in vivo', Journal of Biomechanics, 41(10), pp. 2211-2218.

An, X. C., et al. (2010) 'Anatomic localization of motor entry points and intramuscular nerve endings in the hamstring muscles', Surgical and Radiologic Anatomy, 32(6), pp. 529-537.

Arroyo, E., et al. (2018) 'Effects of supine rest duration on ultrasound measures of the vastus lateralis', Clinical Physiology and Functional Imaging, 38(1), pp. 155-157.

Askling, C. M., et al. (2007) 'Acute First-Time Hamstring Strains during High-Speed Running: A Longitudinal Study Including Clinical and Magnetic Resonance Imaging Findings', The American Journal of Sports Medicine, 35(2), pp. 197-206.

Askling, C. M., et al. (2008) 'Proximal Hamstring Strains of Stretching Type in Different Sports: Injury Situations, Clinical and Magnetic Resonance Imaging Characteristics, and Return to Sport', The American Journal of Sports Medicine, 36(9), pp. 1799-1804.

Askling, C. M., et al. (2013) 'Total proximal hamstring ruptures: clinical and MRI aspects including guidelines for postoperative rehabilitation', Knee surgery, sports traumatology, arthroscopy : official journal of the ESSKA, 21(3), pp. 515.

Baechle, T. R., et al. (2008) Essentials of strength training and conditioning. 3rd edn. Champaign, Ill: Human Kinetics.

Balius, R., et al. (2019) 'Sonographic landmarks in hamstring muscles', Skeletal Radiology, pp. 1-9.

Bamman, M. M., et al. (2000) 'Evaluation of the strength-size relationship in vivo using various muscle size indices', Medicine and Science in Sports and Exercise, 32(7), pp. 1307-1313.

Battermann, N., et al. (2011) 'An Anatomical Study of the Proximal Hamstring Muscle Complex to Elucidate Muscle Strains in this Region', Int J Sports Med, 32(03), pp. 211-215.

Becciolini, M., Bonacchi, G. and Bianchi, S. (2019) 'Ultrasound Features of the Proximal Hamstring Muscle-Tendon-Bone Unit', JOURNAL OF ULTRASOUND IN MEDICINE, 38(5), pp. 1367-1382.

Bejui, J., et al. (1984) 'Anatomical and functional study on the musculus semimembranosus', Anatomia clinica, 6(3), pp. 215-223.

Blazevich, A. J., et al. (2009) 'Anatomical predictors of maximum isometric and concentric knee extensor moment', European Journal of Applied Physiology, 105(6), pp. 869-878.

Blazevich, A. J., Gill, N. D. and Zhou, S. (2006) 'Intra‐ and intermuscular variation in human quadriceps femoris architecture assessed in vivo', Journal of Anatomy, 209(3), pp. 289-310.

Blazevich, A. J. and Sharp, N. C. C. (2006) 'Understanding Muscle Architectural Adaptation: Macro- and Micro-Level Research', Cells Tissues Organs, 181(1), pp. 1-10.

Bolsterlee, B., Gandevia, S. C. and Herbert, R. D. (2016a) 'Effect of Transducer Orientation on Errors in Ultrasound Image-Based Measurements of Human Medial Gastrocnemius Muscle Fascicle Length and Pennation', PLoS ONE, 11(6), pp. e0157273-e0157273.

Bolsterlee, B., Gandevia, S. C. and Herbert, R. D. (2016b) 'Ultrasound imaging of the human medial gastrocnemius muscle: how to orient the transducer so that muscle fascicles lie in the image plane', Journal of Biomechanics, 49(7), pp. 1002-1008.

Bolsterlee, B., et al. (2015) 'Comparison of measurements of medial gastrocnemius architectural parameters from ultrasound and diffusion tensor images', Journal of Biomechanics, 48(6), pp. 1133-1140.

Bourne, M. N., et al. (2017) 'Impact of the Nordic hamstring and hip extension exercises on hamstring architecture and morphology: implications for injury prevention', BRITISH JOURNAL OF SPORTS MEDICINE, 51(5), pp. 469-477.

Brennan, S. F., et al. (2017) 'In vivo fascicle length measurements via B-mode ultrasound imaging with single vs dual transducer arrangements', Journal of Biomechanics, 64, pp. 240-244.

Brockett, C. L., Morgan, D. L. and Proske, U. (2004) 'Predicting hamstring strain injury in elite athletes', MEDICINE AND SCIENCE IN SPORTS AND EXERCISE, 36(3), pp. 379-387.

Carlson, C. (2008) 'The natural history and management of hamstring injuries', Current Reviews in Musculoskeletal Medicine, 1(2), pp. 120-123.

Connell, D. A., et al. (2004) 'Longitudinal Study Comparing Sonographic and MRI Assessments of Acute and Healing Hamstring Injuries', American Journal of Roentgenology, 183(4), pp. 975-984.

Cronin, N. J., et al. (2011) 'Automatic tracking of medial gastrocnemius fascicle length during human locomotion', Journal of Applied Physiology, 111(5), pp. 1491-1496.

Cronin, N. J., et al. (2009) 'Effects of prolonged walking on neural and mechanical components of stretch responses in the human soleus muscle', The Journal of Physiology, 587(17), pp. 4339-4347.

Cronin, N. J. and Lichtwark, G. (2012) 'The use of ultrasound to study muscle–tendon function in human posture and locomotion', Gait & Posture, 37(3), pp. 305-312.

Cohen, S. B., et al. (2011) 'Hamstring Injuries in Professional Football Players: Magnetic Resonance Imaging Correlation with Return to Play', Sports Health: A Multidisciplinary Approach, 3(5), pp. 423-430.

Connell, D. A., et al. (2004) 'Longitudinal Study Comparing Sonographic and MRI Assessments of Acute and Healing Hamstring Injuries', American Journal of Roentgenology, 183(4), pp. 975-984.

De Smet, A. A., et al. (2012) 'MRI appearance of the proximal hamstring tendons in patients with and without symptomatic proximal hamstring tendinopathy', American Journal of Roentgenology, 198(2), pp. 418-422.

Dierckman, B. D. M. D. and Guanche, C. A. M. D. (2012) 'Endoscopic Proximal Hamstring Repair and Ischial Bursectomy', Arthroscopy Techniques, 1(2), pp. e201-e207.

Drazan, J. F., Hullfish, T. J. and Baxter, J. R. (2019) 'An automatic fascicle tracking algorithm quantifying gastrocnemius architecture during maximal effort contractions', PeerJ, 7, pp. e7120-e7120.

e Lima, K. M. M., da Matta, T. T. and de Oliveira, L. F. (2012) 'Reliability of the rectus femoris muscle cross‐sectional area measurements by ultrasonography', Clinical Physiology and Functional Imaging, 32(3), pp. 221-226.

Earp, J. E., et al. (2014) 'The influence of loading intensity on muscle–tendon unit behavior during maximal knee extensor stretch shortening cycle exercise', European Journal of Applied Physiology, 114(1), pp. 59-69.

'<ECR2014 Ultrasound Anatomy.pdf>'.

Ekstrand, J., et al. (2011) 'Epidemiology of Muscle Injuries in Professional Football (Soccer)', The American Journal of Sports Medicine, 39(6), pp. 1226-1232.

Ekstrand, J., et al. (2012) 'Hamstring muscle injuries in professional football: The correlation of MRI findings with return to play', British Journal of Sports Medicine, 46(2), pp. 112-117.

Ekstrand, J., et al. (2016) 'Hamstring injuries have increased by 4% annually in men's professional football, since 2001: A 13-year longitudinal analysis of the UEFA Elite Club injury study', British Journal of Sports Medicine, 50(12), pp. 731-737.

Ernlund, L. and Vieira, L. d. A. (2017) 'Hamstring injuries: update article', Revista Brasileira de Ortopedia (English Edition), 52(4), pp. 373-382.

Feucht, M. J., et al. (2015) 'Gross anatomical and dimensional characteristics of the proximal hamstring origin', Knee Surgery, Sports Traumatology, Arthroscopy, 23(9), pp. 2576-2582.

Franchi, M. V., et al. (2014) 'Architectural, functional and molecular responses to concentric and eccentric loading in human skeletal muscle', Acta Physiologica, 210(3), pp. 642-654.

Franchi, M. V., et al. (2018) 'Muscle thickness correlates to muscle cross‐sectional area in the assessment of strength training‐induced hypertrophy', Scandinavian Journal of Medicine & Science in Sports, 28(3), pp. 846-853.

Freitas, S. R., et al. (2018a) 'Ultrasonographic Measurement of the Biceps Femoris Long-Head Muscle Architecture: Biceps Femoris Architectural Assessment', Journal of Ultrasound in Medicine, 37(4), pp. 977-986.

Freitas, S. R., et al. (2018b) 'Ultrasonographic Measurement of the Biceps Femoris Long‐Head Muscle Architecture', Journal of Ultrasound in Medicine, 37(4), pp. 977-986.

Freitas, S. R., et al. (2018) 'Ultrasonographic Measurement of the Biceps Femoris Long-Head Muscle Architecture: Biceps Femoris Architectural Assessment', Journal of Ultrasound in Medicine, 37(4), pp. 977-986.

Fyfe, J. J., et al. (2013) 'The role of neuromuscular inhibition in hamstring strain injury recurrence', Journal of Electromyography and Kinesiology, 23(3), pp. 523-530.

Gabison, S., et al. (2018) 'Quantitative ultrasound imaging over the ischial tuberosity: An exploratory study to inform tissue health', Journal of Tissue Viability, 27(3), pp. 173-180.

Guanche, C. A. (2015) 'Hamstring injuries', Journal of hip preservation surgery, 2(2), pp. 116-122.

Hägglund, M., et al. (2016) 'Injury recurrence is lower at the highest professional football level than at national and amateur levels: does sports medicine and sports physiotherapy deliver?', British Journal of Sports Medicine, 50(12), pp. 751-758.

Hallén, A., et al. (2014) 'Return to play following muscle injuries in professional footballers', Journal of Sports Sciences, 32(13), pp. 1229-1236.

Hauraix, H., et al. (2017) 'Muscle fascicle shortening behaviour of vastus lateralis during a maximal force–velocity test', European Journal of Applied Physiology, 117(2), pp. 289-299.

Hauraix, H., et al. (2015) 'In vivo maximal fascicle-shortening velocity during plantar flexion in humans', Journal of Applied Physiology, 119(11), pp. 1262-1271.

Hallén, A., et al. (2014) 'Return to play following muscle injuries in professional footballers', Journal of Sports Sciences, 32(13), pp. 1229-1236.

Heiderscheit, B. C., et al. (2010) 'Hamstring strain injuries: Recommendations for diagnosis, rehabilitation, and injury prevention', Journal of Orthopaedic and Sports Physical Therapy, 40(2), pp. 67-81.

Herbert, R. D., et al. (2011) 'In vivo passive mechanical behaviour of muscle fascicles and tendons in human gastrocnemius muscle–tendon units', The Journal of Physiology, 589(21), pp. 5257-5267.

Hickey, J., et al. (2014) 'The financial cost of hamstring strain injuries in the Australian Football League', British Journal of Sports Medicine, 48(8), pp. 729-730.

Ihnatsenka, B. and Boezaart, A. P. (2010) 'Ultrasound: Basic understanding and learning the language', International Journal of Shoulder Surgery, 4(3), pp. 55-62.

John, H. M. B., et al. (2006) 'Incidence, Risk, and Prevention of Hamstring Muscle Injuries in Professional Rugby Union', The American Journal of Sports Medicine, 34(8), pp. 1297-1306.

Kawakami, Y., et al. (2000) 'Architecture of contracting human muscles and its functional significance', Journal of Applied Biomechanics, 16(1), pp. 88-97.

Kellis, E., et al. (2009) 'Validity of architectural properties of the hamstring muscles: Correlation of ultrasound findings with cadaveric dissection', Journal of Biomechanics, 42(15), pp. 2549-2554.

Kellis, E., et al. (2010) 'Muscle architecture variations along the human semitendinosus and biceps femoris (long head) length', Journal of Electromyography and Kinesiology, 20(6), pp. 1237-1243.

Kellis, E., et al. (2012a) 'Architectural differences between the hamstring muscles', J Electromyogr Kinesiol, 22(4), pp. 520-6.

Kellis, E., et al. (2012b) 'In vivo and in vitro Examination of the Tendinous Inscription of the Human Semitendinosus Muscle', Cells Tissues Organs, 195(4), pp. 365-376.

Kerkhoffs, G. M. M. J., et al. (2013) 'Diagnosis and prognosis of acute hamstring injuries in athletes', Knee surgery, sports traumatology, arthroscopy, 21(2), pp. 500-509.

Khoshkhoo, M., et al. (2016) 'The effect of exercise on vastus medialis oblique muscle architecture: An ultrasound investigation', Clinical Anatomy, 29(6), pp. 752-758.

Klein Horsman, M. D., et al. (2006) 'Morphological muscle and joint parameters for musculoskeletal modelling of the lower extremity', Clinical Biomechanics, 22(2), pp. 239-247.

Koulouris, G. and Connell, D. (2003) 'Evaluation of the hamstring muscle complex following acute injury', Skeletal Radiology, 32(10), pp. 582-589.

Koulouris, G., et al. (2007) 'Magnetic Resonance Imaging Parameters for Assessing Risk of Recurrent Hamstring Injuries in Elite Athletes', The American Journal of Sports Medicine, 35(9), pp. 1500-1506.

Kumagai, K., et al. (2000) 'Sprint performance is related to muscle fascicle length in male 100-m sprinters', Journal of Applied Physiology, 88(3), pp. 811-816.

Kumazaki, T., Ehara, Y. and Sakai, T. (2012) 'Anatomy and physiology of hamstring injury', International journal of sports medicine, 33(12), pp. 950-954.

Kwah, L. K., et al. (2013) 'Reliability and validity of ultrasound measurements of muscle fascicle length and pennation in humans: a systematic review', Journal of Applied Physiology, 114(6), pp. 761-769.

Lee, T. C., et al. (1988) 'The site of the tendinous interruption in semitendinosus in man', Journal of Anatomy, 157, pp. 229-231.

Legerlotz, K., Smith, H. K. and Hing, W. A. (2010) 'Variation and reliability of ultrasonographic quantification of the architecture of the medial gastrocnemius muscle in young children', Clinical Physiology and Functional Imaging, 30(3), pp. 198-205.

Lempainen, L., et al. (2015) 'Clinical principles in the management of hamstring injuries', Knee Surgery, Sports Traumatology, Arthroscopy, 23(8), pp. 2449-2456.

Loram, I. D., Maganaris, C. N. and Lakie, M. (2004) 'Paradoxical muscle movement in human standing', The Journal of Physiology, 556(3), pp. 683-689.

Loram, I. D., Maganaris, C. N. and Lakie, M. (2006) 'Use of ultrasound to make noninvasive in vivo measurement of continuous changes in human muscle contractile length', JOURNAL OF APPLIED PHYSIOLOGY, 100(4), pp. 1311-1323.

Martin, B. (1968) 'The origins of the hamstring muscles', Journal of anatomy, 102(Pt 2), pp. 345.

Mason, D., Dickens, V. and Vail, A. (2007) 'Rehabilitation for hamstring injuries: Sports Medicine Update', Scandinavian Journal of Medicine & Science in Sports, 17(2), pp. 191-192.

Mathevon, L. D., et al. (2015) 'Reliability of 2D ultrasound imaging associated with transient ShearWave Elastography method to analyze spastic gastrocnemius medialis muscle architecture and viscoelastic properties', Annals of Physical and Rehabilitation Medicine, 58, pp. e75-e75.

Miyoshi, T., et al. (2008) 'Automatic detection method of muscle fiber movement as revealed by ultrasound images', Medical Engineering and Physics, 31(5), pp. 558-564.

Miller, S. L., Gill, J. and Webb, G. R. (2007) 'The proximal origin of the hamstrings and surrounding anatomy encountered during repair: a cadaveric study', JBJS, 89(1), pp. 44-48.

Narici, M. V., et al. (2003) 'Effect of aging on human muscle architecture', Journal of Applied Physiology, 95(6), pp. 2229-2234.

Noorkoiv, M., et al. (2010) 'In vivo assessment of muscle fascicle length by extended field-of-view ultrasonography', Journal of Applied Physiology, 109(6), pp. 1974-1979.

Neuschwander, T. B., Benke, M. T. and Gerhardt, M. B. (2015) 'Anatomic description of the origin of the proximal hamstring', Arthroscopy: The Journal of Arthroscopic & Related Surgery, 31(8), pp. 1518-1521.

Opar, D. A., Williams, M. D. and Shield, A. J. (2012) 'Hamstring Strain Injuries', Sports Medicine, 42(3), pp. 209-226.

Opar, D. A., et al. (2013) 'Rate of Torque and Electromyographic Development During Anticipated Eccentric Contraction Is Lower in Previously Strained Hamstrings', The American Journal of Sports Medicine, 41(1), pp. 116-125.

Philippon, M. J., et al. (2015) 'A qualitative and quantitative analysis of the attachment sites of the proximal hamstrings', Knee Surgery, Sports Traumatology, Arthroscopy, 23(9), pp. 2554-2561.

Pimenta, R., Blazevich, A. J. and Freitas, S. R. (2018) 'Biceps Femoris Long-Head Architecture Assessed Using Different Sonographic Techniques', Med Sci Sports Exerc, 50(12), pp. 2584-2594.

Potier, T. G., Alexander, C. M. and Seynnes, O. R. (2009) 'Effects of eccentric strength training on biceps femoris muscle architecture and knee joint range of movement', European Journal of Applied Physiology, 105(6), pp. 939-944.

Rab, M., et al. (1997) 'Basic anatomical investigation of semitendinosus and the long head of biceps femoris muscle for their possible use in electrically stimulated neosphincter formation', Surgical and Radiologic Anatomy, 19(5), pp. 287-291.

Raj, I. S., Bird, S. R. and Shield, A. J. (2012) 'Reliability of ultrasonographic measurement of the architecture of the vastus lateralis and gastrocnemius medialis muscles in older adults', Clinical Physiology and Functional Imaging, 32(1), pp. 65-70.

Rana, M., Hamarneh, G. and Wakeling, J. M. (2009) 'Automated tracking of muscle fascicle orientation in B-mode ultrasound images', Journal of Biomechanics, 42(13), pp. 2068-2073.

Rha, D.-W., et al. (2016) 'Intramuscular nerve distribution of the hamstring muscles: Application to treating spasticity: Intramuscular Nerve Patterns of Hamstring Muscles', Clinical Anatomy, 29(6), pp. 746-751.

Ripani, M., et al. (2006) 'The ischiatic region: Normal and MRI anatomy', Journal of Sports Medicine and Physical Fitness, 46(3), pp. 468-475.

Rosenberg, J. G., et al. (2014) 'Reliability of panoramic ultrasound imaging to simultaneously examine muscle size and quality of the medial gastrocnemius', Muscle & Nerve, 49(5), pp. 736-740.

Sato, K., et al. (2012) 'Anatomical study of the proximal origin of hamstring muscles', Journal of Orthopaedic Science, 17(5), pp. 614-618.

Schache, A. G., et al. (2012) 'Mechanics of the human hamstring muscles during sprinting', Medicine and Science in Sports and Exercise, 44(4), pp. 647-658.

Schneider-Kolsky, M. E., et al. (2006) 'A Comparison Between Clinical Assessment and Magnetic Resonance Imaging of Acute Hamstring Injuries', The American Journal of Sports Medicine, 34(6), pp. 1008-1015.

Seidel, G. K., et al. (1996) 'Precise localization of the motor nerve branches to the hamstring muscles: An aid to the conduct of neurolytic procedures', Archives of Physical Medicine and Rehabilitation, 77(11), pp. 1157-1160.

Seymore, K. D., et al. (2017) 'The effect of Nordic hamstring strength training on muscle architecture, stiffness, and strength', European Journal of Applied Physiology, 117(5), pp. 943;1;-11;953;.

Silder, A., et al. (2008) 'MR observations of long-term musculotendon remodeling following a hamstring strain injury', Skeletal Radiology, 37(12), pp. 1101-1109.

Silder, A., et al. (2013) 'Clinical and morphological changes following 2 rehabilitation programs for acute hamstring strain injuries: A randomized clinical trial', Journal of Orthopaedic and Sports Physical Therapy, 43(5), pp. 284-299.

Simpson, C. L., et al. (2017) 'Stretch training induces unequal adaptation in muscle fascicles and thickness in medial and lateral gastrocnemii', Scandinavian Journal of Medicine & Science in Sports, 27(12), pp. 1597-1604.

Stępień, K., et al. (2019) 'Anatomy of proximal attachment, course, and innervation of hamstring muscles: a pictorial essay', Knee Surgery, Sports Traumatology, Arthroscopy, 27(3), pp. 673-684.

Ticinesi, A., et al. (2018) 'Assessing sarcopenia with vastus lateralis muscle ultrasound: an operative protocol', Aging Clinical and Experimental Research, 30(12), pp. 1437-1443.

Timmins, R., et al. (2014) 'BICEPS FEMORIS MUSCLE ARCHITECTURE – THE INFLUENCE OF PREVIOUS INJURY', British Journal of Sports Medicine, 48(7), pp. 665-666.

Timmins, R. G., et al. (2016a) 'Biceps Femoris Architecture and Strength in Athletes with a Previous Anterior Cruciate Ligament Reconstruction', Medicine & Science in Sports & Exercise, 48(3), pp. 337-345.

Timmins, R. G., et al. (2016b) 'Short biceps femoris fascicles and eccentric knee flexor weakness increase the risk of hamstring injury in elite football (soccer): a prospective cohort study', British Journal of Sports Medicine, 50(24), pp. 1524-1535.

Timmins, R. G., et al. (2015) 'Biceps Femoris Long Head Architecture: A Reliability and Retrospective Injury Study', Medicine & Science in Sports & Exercise, 47(5), pp. 905-913.

Tosovic, D., et al. (2016) 'Anatomy of the long head of biceps femoris: An ultrasound study', Clin Anat, 29(6), pp. 738-45.

van der Horst, N., et al. (2015) 'The Preventive Effect of the Nordic Hamstring Exercise on Hamstring Injuries in Amateur Soccer Players : A Randomized Controlled Trial', American Journal of Sports Medicine, 43(6), pp. 1316-1323.

van Dyk, N., et al. (2016) 'Hamstring and Quadriceps Isokinetic Strength Deficits Are Weak Risk Factors for Hamstring Strain Injuries: A 4-Year Cohort Study', American journal of sports medicine, 44(7), pp. 1789-1795.

Varanoske, A. N., et al. (2017) 'Scanning plane comparison of ultrasound‐derived morphological characteristics of the vastus lateralis', Clinical Anatomy, 30(4), pp. 533-542.

Visser, H., et al. 2012. Risk factors of recurrent hamstring injuries: A systematic review.

Vos, R.-J., et al. (2014) 'Clinical findings just after return to play predict hamstring re-injury, but baseline MRI findings do not', British Journal of Sports Medicine: an international peer-reviewed journal of sport and exercise medicine, 48(18), pp. 1377-1384.

Wangensteen, A., et al. (2016) 'Hamstring Reinjuries Occur at the Same Location and Early After Return to Sport: A Descriptive Study of MRI-Confirmed Reinjuries', American journal of sports medicine, 44(8), pp. 2112-2121.

Wickiewicz, T. L., et al. (1983) 'MUSCLE ARCHITECTURE OF THE HUMAN LOWER-LIMB', CLINICAL ORTHOPAEDICS AND RELATED RESEARCH, (179), pp. 275-283.

Woodley, S. J. and Mercer, S. R. (2005) 'Hamstring Muscles: Architecture and Innervation', Cells Tissues Organs, 179(3), pp. 125-141.

Zhao, H. and Zhang, L.-Q. (2011) 'Automatic tracking of muscle fascicles in ultrasound images using localized radon transform', IEEE Transactions on Biomedical Engineering, 58(7), pp. 2094-2101.